# End-to-End Lab Guide

VLANs -> Routing -> DHCP -> DNS -> Apache (HTTP/HTTPS) -> Postfix (Local Delivery) -> Squid Proxy -> UFW Firewall
(Exam-ready, step-by-step, with expected results and troubleshooting)

# 1. What you are building and why the order matters

This guide builds a complete small-organization network and server stack. Each part depends on earlier parts, so we follow the safest, most exam-friendly order:

- Environment and addressing plan (so every device knows where it is).

- VLANs and trunking (so networks are logically separated).

- Inter-VLAN routing (so VLANs can reach the server VLAN).

- DHCP (so clients auto-get correct IP, gateway, and DNS).

- DNS (so names like www.example.com resolve correctly).

- Apache HTTP and two websites (name-based virtual hosting).

- SSL/TLS: private key + CSR workflow, then certificate installation for HTTPS.

- Postfix local delivery (send mail between local users, verify with logs).

- Squid proxy (restricted to your internal networks, with ACLs and logging).

- Firewall (UFW) last: lock down ports after everything works.

Tip: Always validate and test each layer before moving to the next. If you enable the firewall too early, you may block yourself and think services are broken.

# 2. Target topology and IP plan (you can adapt)

Recommended lab topology (Packet Tracer + Ubuntu Server VM):

- 1 switch (Layer 2) with VLAN 10, 20, and 99 (servers).

- 1 router with router-on-a-stick subinterfaces for VLAN 10/20/99.

- 1 Ubuntu Server on VLAN 99 running DHCP, DNS, Apache, Postfix, Squid.

- 2 client PCs: one in VLAN 10 and one in VLAN 20.

Example VLANs and subnets (clean and easy to remember):

| Purpose | VLAN | Subnet | Gateway (router) | Server IP |
| --- | --- | --- | --- | --- |
| Users A | 10 | 10.10.10.0/24 | 10.10.10.254 | DHCP |
| Users B | 20 | 10.10.20.0/24 | 10.10.20.254 | DHCP |
| Servers | 99 | 10.10.99.0/24 | 10.10.99.254 | 10.10.99.10 (static) |

If your Ubuntu VM uses a dual-NIC setup (example enp0s3 NAT + enp0s8 LAN like 192.168.1.12/24), use the LAN-facing IP for clients and services. NAT IP is usually not reachable from other machines without port forwarding.

# 3. Step 1: Switch configuration (VLANs, access ports, trunk)

Goal: create VLANs and ensure the router link is a trunk carrying those VLANs.

## 3.1 Create VLANs

enable

configure terminal

vlan 10

name USERS_A

vlan 20

name USERS_B

vlan 99

name SERVERS

end

write memory

## 3.2 Assign access ports

Example: PC1 on Fa0/2 (VLAN10), PC2 on Fa0/3 (VLAN20), Ubuntu server on Fa0/10 (VLAN99).

configure terminal

interface fa0/2

switchport mode access

switchport access vlan 10

spanning-tree portfast

exit

interface fa0/3

switchport mode access

switchport access vlan 20

spanning-tree portfast

exit

interface fa0/10

switchport mode access

switchport access vlan 99

spanning-tree portfast

exit

end

write memory

## 3.3 Configure trunk to router

configure terminal

interface fa0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,99

exit

end

write memory

## 3.4 Verification (expected output)

show vlan brief

show interfaces trunk

- Expected: Fa0/2 in VLAN10, Fa0/3 in VLAN20, Fa0/10 in VLAN99.

- Expected: Fa0/1 trunking and allowing VLAN 10, 20, 99.

## 3.5 Troubleshooting

- If a port stays in VLAN1: you forgot 'switchport access vlan X' or used the wrong interface.

- If trunk shows none: both sides must support trunking and be set correctly. Recheck Fa0/1 and router interface.

# 4. Step 2: Router-on-a-stick routing (inter-VLAN routing)

Goal: router provides default gateways for each VLAN using 802.1Q subinterfaces.

## 4.1 Enable the physical interface

enable

configure terminal

interface g0/0

no shutdown

end

write memory

## 4.2 Create subinterfaces (gateways)

configure terminal

interface g0/0.10

encapsulation dot1Q 10

ip address 10.10.10.254 255.255.255.0

exit

interface g0/0.20

encapsulation dot1Q 20

ip address 10.10.20.254 255.255.255.0

exit

interface g0/0.99

encapsulation dot1Q 99

ip address 10.10.99.254 255.255.255.0

exit

end

write memory

## 4.3 Verification

show ip interface brief

show running-config interface g0/0.10

show running-config interface g0/0.20

show running-config interface g0/0.99

- Expected: subinterfaces up/up with correct IPs.

- Expected: PCs will use 10.10.10.254 or 10.10.20.254 as their default gateway.

## 4.4 Troubleshooting

- If subinterfaces are down: check the switch trunk VLAN allow list and that the router physical interface is up (no shutdown).

- If clients cannot ping gateways: check VLAN assignment on access ports and trunk configuration.

# 5. Step 3: DHCP server (isc-dhcp-server) with relay

Goal: clients in VLAN10 and VLAN20 automatically receive IP, gateway, and DNS settings. Because DHCP broadcasts do not cross routers, we use ip helper-address (DHCP relay) on the router.

## 5.1 Configure DHCP relay on router (required)

configure terminal

interface g0/0.10

ip helper-address 10.10.99.10

exit

interface g0/0.20

ip helper-address 10.10.99.10

exit

end

write memory

Expected: VLAN10 and VLAN20 clients can get DHCP leases from the server at 10.10.99.10.

## 5.2 Ubuntu server: set static IP (servers VLAN)

On Ubuntu, set a static address. Replace interface name as needed (example ens33 or enp0s8).

ip a

sudo nano /etc/netplan/01-netcfg.yaml

# Example:

# network:

#   version: 2

#   renderer: networkd

#   ethernets:

#     ens33:

#       dhcp4: no

#       addresses: [10.10.99.10/24]

#       gateway4: 10.10.99.254

#       nameservers:

#         addresses: [127.0.0.1, 8.8.8.8]

sudo netplan apply

ip a

ip r

ping -c 3 10.10.99.254

- Expected: the server can ping the router gateway.

- Expected: ip r shows default route via 10.10.99.254.

## 5.3 Install and configure DHCP server

sudo apt update

sudo apt install -y isc-dhcp-server

Bind DHCP to the correct interface:

sudo nano /etc/default/isc-dhcp-server

# Set:

# INTERFACESv4="ens33"

Configure DHCP scopes:

sudo nano /etc/dhcp/dhcpd.conf

default-lease-time 600;

max-lease-time 7200;

authoritative;

option domain-name "amjadweb.com";

option domain-name-servers 10.10.99.10;

subnet 10.10.99.0 netmask 255.255.255.0 {

option routers 10.10.99.254;

}

subnet 10.10.10.0 netmask 255.255.255.0 {

range 10.10.10.100 10.10.10.200;

option routers 10.10.10.254;

option broadcast-address 10.10.10.255;

option domain-name-servers 10.10.99.10;

option domain-name "amjadweb.com";

}

subnet 10.10.20.0 netmask 255.255.255.0 {

range 10.10.20.100 10.10.20.200;

option routers 10.10.20.254;

option broadcast-address 10.10.20.255;

option domain-name-servers 10.10.99.10;

option domain-name "amjadweb.com";

}

Start and verify:

sudo systemctl enable isc-dhcp-server

sudo systemctl restart isc-dhcp-server

sudo systemctl status isc-dhcp-server --no-pager

sudo journalctl -u isc-dhcp-server -n 50 --no-pager

- Expected: service is active (running).

- Expected: no errors like 'Not configured to listen on any interfaces'.

## 5.4 DHCP client test

- Set client NIC to DHCP.

- Expected VLAN10 client: 10.10.10.100-200, gateway 10.10.10.254, DNS 10.10.99.10.

- Expected VLAN20 client: 10.10.20.100-200, gateway 10.10.20.254, DNS 10.10.99.10.

## 5.5 Troubleshooting DHCP (most common)

- No IP on clients: check ip helper-address on router and trunk VLAN allow list on switch.

- DHCP service fails: verify INTERFACESv4 in /etc/default/isc-dhcp-server matches your NIC name (ip a).

- Check leases: /var/lib/dhcp/dhcpd.leases and logs via journalctl.

# 6. Step 4: DNS server (BIND9) - make names work

Goal: resolve your websites and mail hostnames. DNS is required for clean virtual hosting and mail identity.

## 6.1 Install and basic configuration

sudo apt install -y bind9 bind9utils dnsutils

sudo nano /etc/bind/named.conf.options

Recommended internal-lab options (listen on server IP and localhost):

options {

directory "/var/cache/bind";

recursion yes;

allow-query { any; };

listen-on { 127.0.0.1; 10.10.99.10; };

listen-on-v6 { any; };

forwarders {

8.8.8.8;

1.1.1.1;

};

dnssec-validation auto;

};

sudo systemctl restart bind9

sudo systemctl status bind9 --no-pager

sudo ss -ltnup | grep :53

- Expected: bind9 active (running).

- Expected: port 53 listening.

## 6.2 Create a forward zone (amjadweb.com)

sudo nano /etc/bind/named.conf.local

zone "amjadweb.com" {

type master;

file "/etc/bind/db.amjadweb.com";

};

sudo cp /etc/bind/db.local /etc/bind/db.amjadweb.com

sudo nano /etc/bind/db.amjadweb.com

Example zone file with two websites and mail host:

$TTL 86400

@   IN  SOA  ns1.amjadweb.com. admin.amjadweb.com. (

2026021101 ; Serial (increase when you edit)

3600       ; Refresh

900        ; Retry

604800     ; Expire

86400 )    ; Minimum

@    IN  NS   ns1.amjadweb.com.

; A records

ns1  IN  A    10.10.99.10

www  IN  A    10.10.99.10

site2 IN A    10.10.99.10

mail IN  A    10.10.99.10

; MX record for mail delivery to this host

@    IN  MX 10 mail.amjadweb.com.

Validate and restart:

sudo named-checkconf

sudo named-checkzone amjadweb.com /etc/bind/db.amjadweb.com

sudo systemctl restart bind9

## 6.3 DNS testing (server and clients)

dig @127.0.0.1 www.amjadweb.com +short

dig @127.0.0.1 site2.amjadweb.com +short

dig @127.0.0.1 amjadweb.com MX +short

- Expected: www and site2 resolve to 10.10.99.10.

- Expected: MX returns mail.amjadweb.com.

On clients, ensure DNS server is set to 10.10.99.10 (from DHCP). Then:

nslookup www.amjadweb.com

ping -c 2 www.amjadweb.com

## 6.4 Troubleshooting DNS

- SERVFAIL: zone file error or serial format issue. Run named-checkzone.

- No answer: client DNS not pointing to your server. Check DHCP option domain-name-servers.

- Not listening on LAN IP: verify listen-on in named.conf.options and restart bind9.

# 7. Step 5: Apache web server (HTTP) with two websites (name-based hosting)

Goal: host two different websites on the same server IP using ServerName/ServerAlias.

## 7.1 Install and enable Apache

sudo apt install -y apache2

sudo systemctl enable --now apache2

sudo systemctl status apache2 --no-pager

sudo ss -ltnp | grep :80

## 7.2 Create two website folders and pages

sudo mkdir -p /var/www/www.amjadweb.com

sudo mkdir -p /var/www/site2.amjadweb.com

echo "<h1>Site 1 - www.amjadweb.com</h1><p>HTTP works.</p>" | sudo tee /var/www/www.amjadweb.com/index.html

echo "<h1>Site 2 - site2.amjadweb.com</h1><p>HTTP works.</p>" | sudo tee /var/www/site2.amjadweb.com/index.html

### Optional: full HTML content (multi-line) for both websites

If you want richer pages than the one-line 'echo' commands above, use these heredoc blocks to write complete HTML files (recommended for the exam because it shows you understand where web content lives).

#### Site 1 (www.amjadweb.com):

sudo tee /var/www/www.amjadweb.com/index.html > /dev/null <<'HTML'

<!doctype html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>Site 1 - www.amjadweb.com</title>

</head>

<body>

<h1>Site 1 - www.amjadweb.com</h1>

<p>HTTP works ✅</p>

<p>Server: <strong>Apache</strong></p>

<ul>

<li>VirtualHost based on hostname (name-based hosting)</li>

<li>DocumentRoot: /var/www/www.amjadweb.com</li>

</ul>

<hr>

<p><small>Generated in the Systems & Network Administration lab.</small></p>

</body>

</html>

HTML

#### Site 2 (site2.amjadweb.com):

sudo tee /var/www/site2.amjadweb.com/index.html > /dev/null <<'HTML'

<!doctype html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>Site 2 - site2.amjadweb.com</title>

</head>

<body>

<h1>Site 2 - site2.amjadweb.com</h1>

<p>HTTP works ✅</p>

<p>This is a different website served from a different DocumentRoot.</p>

<ul>

<li>DocumentRoot: /var/www/site2.amjadweb.com</li>

<li>Same server IP, different hostname</li>

</ul>

<hr>

<p><small>Generated in the Systems & Network Administration lab.</small></p>

</body>

</html>

HTML

#### Reload Apache after editing content:

sudo systemctl reload apache2

sudo chown -R www-data:www-data /var/www/www.amjadweb.com /var/www/site2.amjadweb.com

## 7.3 Create VirtualHost configs (HTTP)

sudo nano /etc/apache2/sites-available/www.amjadweb.com.conf

<VirtualHost *:80>

ServerName www.amjadweb.com

DocumentRoot /var/www/www.amjadweb.com

ErrorLog ${APACHE_LOG_DIR}/www.amjadweb.com_error.log

CustomLog ${APACHE_LOG_DIR}/www.amjadweb.com_access.log combined

</VirtualHost>

sudo nano /etc/apache2/sites-available/site2.amjadweb.com.conf

<VirtualHost *:80>

ServerName site2.amjadweb.com

DocumentRoot /var/www/site2.amjadweb.com

ErrorLog ${APACHE_LOG_DIR}/site2.amjadweb.com_error.log

CustomLog ${APACHE_LOG_DIR}/site2.amjadweb.com_access.log combined

</VirtualHost>

Enable sites and reload:

sudo a2ensite www.amjadweb.com.conf

sudo a2ensite site2.amjadweb.com.conf

sudo a2dissite 000-default.conf

sudo apache2ctl configtest

sudo systemctl reload apache2

## 7.4 Test HTTP

curl -I http://www.amjadweb.com

curl -I http://site2.amjadweb.com

- Expected: HTTP/1.1 200 OK (or 301 if you later redirect).

- Expected: correct site content based on hostname.

## 7.5 Troubleshooting Apache HTTP

- DNS not resolving hostnames: fix BIND9 or client DNS settings first.

- Wrong website shown: check ServerName in each VirtualHost and ensure both configs are enabled.

- Syntax errors: run 'apache2ctl configtest' and check /var/log/apache2/*.log.

# 8. Step 6: SSL/TLS certification - private key + CSR workflow + HTTPS

Goal: build HTTPS correctly using the standard workflow: private key -> CSR -> certificate -> Apache SSL VirtualHost.

## 8.1 Key concepts (exam important)

- Private key: secret file kept on the server. It proves identity and is used to decrypt TLS key exchange.

- CSR (Certificate Signing Request): contains your public key and identity (CN, organization). You send the CSR to a CA.

- Certificate (CRT): signed by a CA (or self-signed in lab). It binds your domain name to your public key.

- HTTPS: HTTP over TLS. Uses port 443.

## 8.2 Create private keys (one per site)

sudo mkdir -p /etc/ssl/lab

sudo chmod 700 /etc/ssl/lab

sudo openssl genpkey -algorithm RSA -pkeyopt rsa_keygen_bits:2048 \

-out /etc/ssl/lab/www.amjadweb.com.key

sudo openssl genpkey -algorithm RSA -pkeyopt rsa_keygen_bits:2048 \

-out /etc/ssl/lab/site2.amjadweb.com.key

sudo chmod 600 /etc/ssl/lab/*.key

- Expected: two .key files created and readable only by root.

- Troubleshoot permission errors: keep keys in a root-only directory and set 600 permissions.

## 8.3 Create CSRs (standard workflow)

sudo openssl req -new \

-key /etc/ssl/lab/www.amjadweb.com.key \

-out /etc/ssl/lab/www.amjadweb.com.csr \

-subj "/C=LK/ST=NA/L=NA/O=LAB/OU=IT/CN=www.amjadweb.com"

sudo openssl req -new \

-key /etc/ssl/lab/site2.amjadweb.com.key \

-out /etc/ssl/lab/site2.amjadweb.com.csr \

-subj "/C=LK/ST=NA/L=NA/O=LAB/OU=IT/CN=site2.amjadweb.com"

Inspect a CSR (verify CN is correct):

openssl req -in /etc/ssl/lab/www.amjadweb.com.csr -noout -text | grep -E "Subject:|CN="

## 8.4 Get certificates: two options

### Option A: Self-signed certificates (lab safe)

This is acceptable for labs and internal testing. Browsers will show a warning because it is not signed by a public CA.

sudo openssl x509 -req -days 365 \

-in /etc/ssl/lab/www.amjadweb.com.csr \

-signkey /etc/ssl/lab/www.amjadweb.com.key \

-out /etc/ssl/lab/www.amjadweb.com.crt

sudo openssl x509 -req -days 365 \

-in /etc/ssl/lab/site2.amjadweb.com.csr \

-signkey /etc/ssl/lab/site2.amjadweb.com.key \

-out /etc/ssl/lab/site2.amjadweb.com.crt

### Option B: CA-signed certificate (real world)

- You generate CSR on your server.

- You submit CSR to a Certificate Authority (CA).

- CA returns a signed certificate (and sometimes intermediate chain).

- You install the certificate and chain in Apache.

- For a public CA (Lets Encrypt), you usually use certbot (not required for this lab unless your exam specifies it).

## 8.5 Configure Apache for HTTPS (two SSL VirtualHosts)

sudo a2enmod ssl

sudo systemctl restart apache2

Create SSL VirtualHost for www.amjadweb.com:

sudo nano /etc/apache2/sites-available/www.amjadweb.com-ssl.conf

<VirtualHost *:443>

ServerName www.amjadweb.com

DocumentRoot /var/www/www.amjadweb.com

SSLEngine on

SSLCertificateFile /etc/ssl/lab/www.amjadweb.com.crt

SSLCertificateKeyFile /etc/ssl/lab/www.amjadweb.com.key

ErrorLog ${APACHE_LOG_DIR}/www.amjadweb.com_ssl_error.log

CustomLog ${APACHE_LOG_DIR}/www.amjadweb.com_ssl_access.log combined

</VirtualHost>

Create SSL VirtualHost for site2.amjadweb.com:

sudo nano /etc/apache2/sites-available/site2.amjadweb.com-ssl.conf

<VirtualHost *:443>

ServerName site2.amjadweb.com

DocumentRoot /var/www/site2.amjadweb.com

SSLEngine on

SSLCertificateFile /etc/ssl/lab/site2.amjadweb.com.crt

SSLCertificateKeyFile /etc/ssl/lab/site2.amjadweb.com.key

ErrorLog ${APACHE_LOG_DIR}/site2.amjadweb.com_ssl_error.log

CustomLog ${APACHE_LOG_DIR}/site2.amjadweb.com_ssl_access.log combined

</VirtualHost>

Enable SSL sites and reload:

sudo a2ensite www.amjadweb.com-ssl.conf

sudo a2ensite site2.amjadweb.com-ssl.conf

sudo apache2ctl configtest

sudo systemctl reload apache2

sudo ss -ltnp | grep :443

## 8.6 Optional: Redirect HTTP -> HTTPS

For each HTTP VirtualHost, add a redirect so users always use HTTPS.

# Example inside the <VirtualHost *:80> block:

Redirect permanent / https://www.amjadweb.com/

## 8.7 HTTPS verification

curl -kI https://www.amjadweb.com

curl -kI https://site2.amjadweb.com

openssl s_client -connect www.amjadweb.com:443 -servername www.amjadweb.com </dev/null | grep -E "subject=|issuer="

- Expected: curl shows 200 OK (use -k for self-signed).

- Expected: openssl s_client shows subject CN=www.amjadweb.com.

## 8.8 Troubleshooting HTTPS (common)

- Apache fails to start after enabling SSL: check 'apache2ctl configtest' and SSL file paths.

- Wrong certificate served: ensure each SSL VirtualHost has correct ServerName and correct certificate file.

- Permission denied reading key: keep keys root-only and ensure Apache can read them. In many Ubuntu setups, /etc/ssl/private uses ssl-cert group; otherwise store under /etc/ssl/lab with correct permissions and test.

- Browser still shows old cert: clear cache or restart browser; increase serial if using internal CA.

# 9. Step 7: Mail server (Postfix local delivery)

Goal: send mail between local users on the same machine, confirm delivery via /var/mail and logs. This is safe and exam-friendly.

## 9.1 Install Postfix + mailutils

sudo apt install -y postfix mailutils

sudo systemctl enable --now postfix

sudo systemctl status postfix --no-pager

## 9.2 Set identity (myhostname) and limits

sudo hostnamectl set-hostname mail.amjadweb.com

sudo postconf -e "myhostname = mail.amjadweb.com"

sudo postconf -e "mailbox_size_limit = 0"

sudo postconf -e "message_size_limit = 10240000"

sudo systemctl restart postfix

postconf myhostname mailbox_size_limit message_size_limit

## 9.3 Create test users (if needed)

sudo adduser amjad

sudo adduser homeuser

## 9.4 Send a local test email and verify delivery

echo "Hello from Postfix at $(date)" | mail -s "TEST MAIL" homeuser

sudo tail -n 50 /var/log/mail.log

ls -l /var/mail/homeuser

Read mail as the user:

su - homeuser

mail

# q = quit and save messages to ~/mbox (and empties /var/mail/homeuser)

# x = exit without saving changes

## 9.5 Redirect root mail (aliases)

sudo nano /etc/aliases

# Add:

# root: amjad

sudo newaliases

sudo systemctl reload postfix

## 9.6 Troubleshooting Postfix local delivery

- Mail not delivered: check /var/log/mail.log for status=sent or errors.

- No /var/mail/username: ensure the user exists (getent passwd homeuser).

- Mail disappears after reading: you used q, which saved to ~/mbox and cleared the spool. Use x if you want to keep it in /var/mail.

# 10. Step 8: Squid proxy (restricted, safe, exam-ready)

Goal: Squid listens on 3128, allows only your internal networks, blocks certain sites during office hours, and denies all else.

## 10.1 Install and start

sudo apt install -y squid

sudo systemctl enable --now squid

sudo systemctl status squid --no-pager

## 10.2 Clean configuration (no duplicate http_port)

Always ensure there is only one http_port 3128 line. Duplicate ports cause '(98) Address already in use'.

sudo grep -RIn '^\s*http_port\b' /etc/squid/squid.conf /etc/squid/conf.d/

Edit config:

sudo cp /etc/squid/squid.conf /etc/squid/squid.conf.bak.$(date +%F-%H%M)

sudo nano /etc/squid/squid.conf

Recommended ACL block (adjust networks):

# Bind to server IP in the client-facing network (recommended)

http_port 10.10.99.10:3128

# Allowed client networks

acl allowed_clients src 10.10.10.0/24

acl allowed_clients src 10.10.20.0/24

# Block list and time rules

acl blocksite dstdomain "/etc/squid/blocksite"

acl office_hours time MTWHF 08:00-17:00

# Safe ports and CONNECT

acl Safe_ports port 80 443

acl CONNECT method CONNECT

http_access deny !Safe_ports

http_access deny CONNECT !Safe_ports

# Block listed sites during office hours

http_access deny blocksite office_hours

# Allow our clients and localhost

http_access allow allowed_clients

http_access allow localhost

# IMPORTANT: close proxy to everyone else

http_access deny all

## 10.3 Create blocklist file

sudo nano /etc/squid/blocksite

# Example:

.facebook.com

.youtube.com

## 10.4 Validate and restart

sudo squid -k parse

sudo systemctl restart squid

sudo systemctl status squid --no-pager

sudo ss -ltnp | grep 3128

## 10.5 Proxy testing

On the server (local test):

curl -I -x http://127.0.0.1:3128 http://example.com

sudo tail -n 20 /var/log/squid/access.log

From a client PC (replace server IP):

curl -I -x http://10.10.99.10:3128 http://example.com

## 10.6 Troubleshooting Squid

- Address already in use: duplicate http_port. Search and remove duplicates, then restart.

- Invalid ACL type: ensure syntax is 'acl NAME src NET/MASK' (do not insert extra words).

- 403 TCP_DENIED: the client IP is not in allowed_clients. Add correct subnet.

- Nothing in access.log: traffic is not reaching Squid (routing, VLAN, firewall).

# 11. Step 9: UFW firewall (apply after everything works)

Goal: keep the server secure by allowing only necessary ports. Always allow SSH first to avoid locking yourself out.

## 11.1 Enable UFW safely

sudo ufw allow OpenSSH

sudo ufw default deny incoming

sudo ufw default allow outgoing

## 11.2 Allow required services

# DNS

sudo ufw allow 53/tcp

sudo ufw allow 53/udp

# Web

sudo ufw allow 80/tcp

sudo ufw allow 443/tcp

# Mail (local delivery does not require open ports, but allow if testing SMTP)

sudo ufw allow 25/tcp

sudo ufw allow 587/tcp

# Squid proxy - restrict to internal subnets only

sudo ufw allow from 10.10.10.0/24 to any port 3128 proto tcp

sudo ufw allow from 10.10.20.0/24 to any port 3128 proto tcp

Enable and check status:

sudo ufw enable

sudo ufw status numbered

sudo ufw status verbose

## 11.3 Troubleshooting firewall

- Service works before UFW, then breaks after enabling: UFW is blocking. Add allow rules for the correct port/protocol and restart the service if needed.

- Check listening ports: 'sudo ss -ltnup' to confirm services are actually listening.

- If SSH locked out: use console access to disable UFW (sudo ufw disable) then fix rules.

# 12. Final end-to-end verification (exam checklist)

## 12.1 Services running

systemctl status isc-dhcp-server bind9 apache2 postfix squid --no-pager

## 12.2 Ports listening

sudo ss -ltnup | grep -E ':53|:67|:80|:443|:25|:587|:3128'

## 12.3 DNS

dig @127.0.0.1 www.amjadweb.com +short

dig @127.0.0.1 site2.amjadweb.com +short

dig @127.0.0.1 amjadweb.com MX +short

## 12.4 Web (HTTP/HTTPS)

curl -I http://www.amjadweb.com

curl -kI https://www.amjadweb.com

curl -I http://site2.amjadweb.com

curl -kI https://site2.amjadweb.com

## 12.5 Mail local delivery

echo "Exam mail test $(date)" | mail -s "MAIL TEST" homeuser

sudo tail -n 30 /var/log/mail.log

ls -l /var/mail/homeuser

## 12.6 Squid proxy

curl -I -x http://127.0.0.1:3128 http://example.com

sudo tail -n 20 /var/log/squid/access.log

## 12.7 UFW

sudo ufw status numbered

# 13. High-yield exam notes (quick memory)

- VLAN: separates broadcast domains on the same switch. Access port = one VLAN. Trunk = multiple VLANs with 802.1Q tags.

- Router-on-a-stick: one router interface with multiple subinterfaces; each subinterface is a VLAN gateway.

- DHCP relay (ip helper-address): needed because DHCP Discover is broadcast and routers do not forward broadcasts by default.

- DNS A record maps name to IP. MX record tells where mail for the domain should go.

- Virtual hosting: multiple websites on one IP using different Host headers (ServerName).

- HTTPS: HTTP over TLS. Key + CSR + certificate. Private key stays secret on server.

- Postfix local delivery: mail delivered to /var/mail/username; the mail program may save to ~/mbox when you quit with q.

- Squid: ACL order matters. Deny all at the end. Restrict by source networks to avoid open proxy.

- Firewall: enable last, allow only required ports. Always allow SSH first.

# Appendix A — How to answer past-paper / in‑course questions (patterns)

Use this appendix when a question says: “Explain”, “Compare”, “Show steps”, “Give commands + expected output”, or “Troubleshoot”. Most marking schemes give marks for: (1) correct order, (2) correct commands, (3) correct expected observations, and (4) a short reason for each step.

## A1. Universal answer pattern (works for DNS/Apache/Postfix/Squid/UFW)

State the goal in one line (what you are trying to achieve).

State the assumptions (IP address, domain name, interface, VLAN/subnet).

Show the configuration change (file path + the exact lines).

Validate the configuration (syntax check command).

Restart/reload the service.

Verify externally (client-side test) + verify internally (logs).

If it fails: show troubleshooting commands in this order: service status → logs → ports → config validation → firewall → routing/DNS.

## A2. Common short theory blocks examiners expect

These are the short definitions/contrasts that often appear in theory questions.

### Address types (IP vs MAC vs Port vs Socket)

IP address (Layer 3): identifies a host/interface in an IP network (changes when you move networks).

MAC address (Layer 2): identifies a network interface on a LAN; used for Ethernet delivery inside the same LAN.

Port (Layer 4): identifies an application/service on a host (e.g., 80 HTTP, 443 HTTPS, 25 SMTP).

Socket: IP + Port + Protocol (TCP/UDP). Example: 192.168.1.12:3128/TCP is a socket used by Squid.

### Network devices (hub vs switch vs router)

Hub: Layer 1; repeats bits to all ports; one collision domain; rarely used today.

Switch: Layer 2; forwards frames using MAC table; each port is separate collision domain; same broadcast domain unless VLANs.

Router: Layer 3; forwards packets between different IP networks; separates broadcast domains; default gateway for hosts.

### VLAN + Router-on-a-stick (why it is used)

VLAN: logically splits one physical switch into multiple broadcast domains (e.g., VLAN10, VLAN20).

Trunk port: carries multiple VLANs between switch ↔ router/switch (802.1Q tags).

Access port: carries one VLAN (untagged) to end devices.

Router-on-a-stick: one router physical interface + multiple subinterfaces (e.g., G0/0.10, G0/0.20) each with a gateway IP for that VLAN.

Why: cheaper than multiple router interfaces; common in labs and small networks.

### DNS basics (A, CNAME, MX, NS, SOA)

A record: name → IPv4 address (www.example.com → 192.168.1.12).

CNAME: alias name → canonical name (blog.example.com → www.example.com).

MX: mail exchanger for a domain (example.com MX mail.example.com).

NS: which name server is authoritative for the zone.

SOA: start of authority; contains serial number and zone timing parameters.

### Apache vs Nginx (common compare question)

Apache: process/thread-based model; very flexible; rich module ecosystem; .htaccess per-directory overrides.

Nginx: event-driven, non-blocking; very efficient for high concurrency; often used as reverse proxy/load balancer in front of app servers.

Both support virtual hosting, TLS, reverse proxying. Choice depends on workload, features, and admin preference.

### SSL/TLS + certificates (what’s a private key, CSR, certificate?)

Private key: secret key stored on the server; used to prove identity and decrypt key exchange in some TLS modes.

Public key: can be shared; derived from private key; embedded in CSR/cert.

CSR (Certificate Signing Request): request containing subject info + public key; signed by your private key; sent to a CA.

Certificate: CA-signed document binding a domain name to a public key (includes validity period + CA signature).

Chain of trust: server cert → intermediate CA → root CA.

### Email basics (Postfix local delivery)

MTA (Mail Transfer Agent): Postfix; receives and routes mail (SMTP).

Local delivery: mail to local users stored in /var/mail/<user> (mbox) or Maildir (depending config).

Aliases: /etc/aliases can redirect mail (e.g., root → amjad); apply with newaliases.

### Proxy + firewall (Squid + UFW)

Squid: forward proxy; clients send HTTP requests to Squid; Squid fetches from the internet and can cache/filter.

ACL order matters: Squid uses first-match logic; always end with 'http_access deny all' to avoid an open proxy.

UFW: host firewall; set default deny incoming; allow only required ports; restrict proxy port to internal subnets.

# Appendix B — Lab 01/02/03 answer patterns (commands + expected output + troubleshooting)

## Lab 01 — Basic network checks + DNS + web access (pattern answers)

Use these steps when the question asks you to show how you verified network connectivity and name resolution.

### B1. Check interfaces and IP addresses

ip a

ip r

Expected:

You see your LAN-facing interface IP (e.g., 192.168.1.12/24 or 10.10.99.10/24).

You see a default route (default via <gateway> dev <iface>).

Troubleshoot if missing IP/default route:

sudo systemctl status systemd-networkd NetworkManager --no-pager

sudo netplan get

sudo netplan try

sudo netplan apply

nmcli dev status 2>/dev/null || true

### B2. Test connectivity to gateway + internet

ping -c 4 <GATEWAY_IP>

ping -c 4 8.8.8.8

Expected:

Gateway ping replies (means LAN link + gateway reachable).

8.8.8.8 replies (means routing/NAT to internet works).

If 8.8.8.8 fails but gateway works: check router NAT or upstream internet. If gateway fails: check cabling/VLAN/port config.

### B3. Test DNS resolution (server and client)

# Query your own DNS server (BIND) directly

dig @<DNS_SERVER_IP> www.<DOMAIN> +short

dig @<DNS_SERVER_IP> <DOMAIN> MX +short

# Test via system resolver (uses /etc/resolv.conf or systemd-resolved)

getent hosts www.<DOMAIN>

Expected:

dig returns the correct A record IP (your web server).

MX query returns your mail host (mail.<DOMAIN>).

getent hosts resolves the name using your system DNS settings.

Troubleshooting:

sudo named-checkconf

sudo named-checkzone <DOMAIN> /etc/bind/db.<DOMAIN>

sudo systemctl status bind9 --no-pager

sudo journalctl -u bind9 --no-pager | tail -n 80

sudo ss -ltnup | grep :53

### B4. Test web server (HTTP/HTTPS)

curl -I http://www.<DOMAIN>

curl -k -I https://www.<DOMAIN>

Expected:

HTTP returns 200/301 and Server: Apache/…

HTTPS returns 200/301 (with -k ignoring self-signed warning).

Troubleshooting:

sudo apache2ctl configtest

sudo systemctl status apache2 --no-pager

sudo apache2ctl -S

sudo tail -n 80 /var/log/apache2/error.log

sudo ss -ltnp | grep -E ':80|:443'

## Lab 02 — Linux host management patterns (files, processes, users)

These patterns help you answer questions about basic host administration.

### B5. File/dir operations + ownership

ls -la

mkdir -p lab2/testdir

touch lab2/testdir/file1

cp lab2/testdir/file1 lab2/testdir/file1.copy

mv lab2/testdir/file1.copy lab2/testdir/file2

rm -f lab2/testdir/file2

Expected: ls shows new directories/files; cp duplicates; mv renames; rm removes.

### B6. Permissions (chmod/chown/chgrp) — common answer format

# show permissions

ls -l <FILE>

# change owner + group

sudo chown <USER>:<GROUP> <FILE>

# numeric permissions example: rwxr-x---

chmod 750 <FILE>

# symbolic example: add execute for owner

chmod u+x <FILE>

# add user to group

sudo usermod -aG <GROUP> <USER>

newgrp <GROUP>   # re-login or use newgrp to apply

Expected: ls -l reflects permission/owner changes. Group membership applies after re-login/newgrp.

### B7. Processes/services (systemctl) — exam pattern

systemctl status <service> --no-pager

journalctl -u <service> --no-pager | tail -n 80

Expected: status shows active/running or failing; logs show reason if failing.

## Lab 03 — Permissions + special bits (SUID/SGID/Sticky) patterns

### B8. Special permission bits

SUID (set-user-ID): executable runs as file owner (shown as 's' in user execute position). chmod u+s file or chmod 4755 file.

SGID (set-group-ID): executable runs as file group; on directories, new files inherit the directory group. chmod g+s dir or chmod 2755 dir.

Sticky bit: on directories (e.g., /tmp), only owner can delete own files. chmod +t dir or chmod 1777 dir.

# create a test directory

mkdir -p ~/lab3/stickytest

chmod 1777 ~/lab3/stickytest

ls -ld ~/lab3/stickytest

# SUID demo (do NOT set suid on arbitrary binaries in real life)

cp /usr/bin/passwd ~/lab3/passwd_copy

chmod 4755 ~/lab3/passwd_copy

ls -l ~/lab3/passwd_copy

Expected: ls shows 't' on sticky directory; SUID binary shows 's' in user execute field.

### B9. umask (default permissions) pattern

umask

umask 022

touch umask_test_file

mkdir umask_test_dir

ls -l umask_test_file

ls -ld umask_test_dir

Expected: with umask 022, new files are typically 644 and directories 755 (depends on program defaults).

# Appendix C — Quick exam-style questions + model answers (high‑yield)

Q: Explain why DNS must be configured before Apache virtual hosting.

A: Because browsers resolve names to IPs before connecting. Virtual hosting also uses Host header / ServerName; if the client cannot resolve the name to your server IP, you cannot test or access the correct site.

Q: What is name-based virtual hosting in Apache? How is it related to one IP hosting multiple websites?

A: Name-based virtual hosting means Apache chooses the site based on the HTTP Host header (ServerName/ServerAlias). Multiple domains can point to the same IP, and Apache serves different DocumentRoots depending on the requested hostname.

Q: What is a CSR and why do we generate it?

A: CSR (Certificate Signing Request) contains your subject details + public key and is signed by your private key. You send it to a CA so the CA can verify and issue a certificate that browsers trust.

Q: Why must Squid configuration end with 'http_access deny all'?

A: To avoid an open proxy. If you forget the final deny, any IP may use your proxy to access the internet (security risk, abuse).

Q: Why enable UFW only after services are working?

A: If you enable firewall too early, you may block needed ports and waste time debugging. Best practice: verify services → then lock down ports → then re-test.

Q: Why does /var/mail/<user> sometimes become 0 bytes after reading mail with the 'mail' command?

A: When you quit mail with 'q', it saves messages into ~/mbox and truncates the system spool file /var/mail/<user>. Use 'x' to exit without moving mail.

## C2. Mini checklists (what to show as proof in practical exams)

DNS: named-checkzone OK + dig returning A/MX + bind9 service running + :53 listening.

Apache: apache2ctl configtest OK + curl -I http://... + ss shows :80/:443 + correct vhost shown in apache2ctl -S.

TLS: show private key exists + CSR created + certificate details (openssl x509 -in cert -noout -text) + HTTPS works in curl/browser.

Postfix: send a local mail + show /var/log/mail.log delivered + mailq empty + mailbox file updated.

Squid: squid -k parse clean + ss shows :3128 + curl with -x proxy works + access.log has client IP + ACL denies unauthorized subnet.

UFW: status active + rules numbered + only required ports open + proxy port restricted to internal subnet.

# Appendix D — Extra theory blocks (ICANN/IANA/APNIC, RJ45, AS/BGP, subnetting)

## D1. ICANN, IANA, RIRs, APNIC, ISP vs Telco

ICANN (Internet Corporation for Assigned Names and Numbers): coordinates global DNS name system and IP address allocation policies (overall coordination).

IANA (Internet Assigned Numbers Authority): maintains central registries (root zone, IP blocks, protocol parameters). Operated under ICANN.

RIR (Regional Internet Registry): allocates IP address blocks within a region (e.g., APNIC for Asia-Pacific).

APNIC (Asia-Pacific Network Information Centre): RIR for Asia-Pacific; allocates IPv4/IPv6 and AS numbers to ISPs/organizations in the region.

ISP (Internet Service Provider): sells internet connectivity to users/businesses; may provide IP addresses, DNS, email, hosting, etc.

Telco: telecommunications company providing physical network infrastructure (fiber/copper/mobile). Sometimes also acts as an ISP, but 'telco' emphasizes the carrier infrastructure.

Common exam pattern: “Explain the chain from ICANN → IANA → RIR (APNIC) → ISP → customer.”

## D2. Global internet connectivity: AS + BGP (how data travels)

AS (Autonomous System): a large network (ISP, company, university) under one administrative control; identified by an AS number (ASN).

BGP (Border Gateway Protocol): routing protocol used between ASes on the internet; exchanges reachable prefixes and chooses paths based on policy.

Typical flow: Your PC → default gateway → ISP access network → ISP core → upstream/peering links → destination AS → destination network.

Why BGP matters: the internet is not a single network; it is many ASes connected by peering/transit; BGP is the “glue” that decides where to send traffic.

## D3. RJ45 cabling essentials (often asked)

RJ45: common connector for Ethernet twisted-pair cables.

Straight-through cable: both ends use the same standard (T568A-A or T568B-B). Used for host↔switch, switch↔router (most modern devices support auto‑MDI/MDIX anyway).

Crossover cable: one end T568A and the other T568B. Traditionally used for like-to-like connections (switch↔switch, PC↔PC) if no auto‑MDI/MDIX.

T568A vs T568B: two pinout standards; B is common in many labs. Key is consistency on both ends for straight-through.

## D4. Subnetting quick pattern (how to show steps)

When asked to calculate subnet mask / network / broadcast / usable range, show these steps:

Write the prefix length (e.g., /24) and convert to mask (255.255.255.0).

Find block size in the changing octet: block = 256 − maskOctet.

Network address: round the IP down to nearest block.

Broadcast: network + (block − 1).

Usable hosts: network+1 to broadcast−1.

Usable count: 2^(hostBits) − 2.

Example: 192.168.1.12/24

Mask = 255.255.255.0

Network = 192.168.1.0

Broadcast = 192.168.1.255

Usable = 192.168.1.1 – 192.168.1.254

Hosts = 2^(32-24) - 2 = 254

## D5. Quick OpenSSL lab theory (encoding vs hashing vs encryption)

Encoding: reversible representation change (Base64). Not security.

Hashing: one-way digest (SHA-256). Used for integrity checks and password storage (with salt).

Symmetric encryption: same secret key encrypts/decrypts (AES). Fast.

Asymmetric encryption: public/private key pair (RSA/ECDSA). Used for key exchange and signatures.

TLS uses both: asymmetric for authentication/key exchange + symmetric for bulk data.