VLANs to Firewall: End-to-End Practical Setup Guide

VLANs -> Routing -> DHCP -> DNS -> Apache (HTTP/HTTPS) -> Squid Proxy -> Firewall

Systems and Network Administration - Lab Build (Cisco/Packet Tracer + Ubuntu Server)

# 1. Goal and What You Will Build

This guide builds a complete small-organization network from scratch. You will create VLANs on a switch, route between VLANs using a router-on-a-stick setup, assign IP addresses automatically with DHCP, resolve names with DNS, host a website with Apache (HTTP and HTTPS), enforce browsing policy with a Squid proxy, and secure everything with a firewall.

# 2. Target Topology and Address Plan

Devices:

Layer-2 switch (Cisco 2960 style)

Router (Cisco router) for inter-VLAN routing (router-on-a-stick)

Ubuntu Server (DHCP + DNS + Apache + Squid)

Two client PCs/VMs (one in VLAN10, one in VLAN20)

Suggested cabling/ports (adjust to your lab):

| Link | Switch Port | Notes |
| --- | --- | --- |
| Router <-> Switch | Fa0/1 | Trunk carrying VLAN 10, 20, 99 |
| PC1 (Users A) | Fa0/2 | Access port in VLAN 10 |
| PC2 (Users B) | Fa0/3 | Access port in VLAN 20 |
| Ubuntu Server | Fa0/10 | Access port in VLAN 99 (Servers) |

Address plan (simple and standard):

| VLAN | Subnet | Gateway (Router) | Server/Notes |
| --- | --- | --- | --- |
| 10 (USERS_A) | 10.10.10.0/24 | 10.10.10.254 | Clients via DHCP (10.10.10.100-200) |
| 20 (USERS_B) | 10.10.20.0/24 | 10.10.20.254 | Clients via DHCP (10.10.20.100-200) |
| 99 (SERVERS) | 10.10.99.0/24 | 10.10.99.254 | Ubuntu server: 10.10.99.10 (static) |

# 3. Build Order (Why This Order)

Follow this order because each layer depends on the previous one:
1) VLANs/trunking must exist before routing works.
2) Routing must work before DHCP relay can serve multiple VLANs.
3) Clients must get IP + DNS settings (DHCP) before they can use DNS names.
4) DNS should be ready before you test web hosting by name (www.lab.local).
5) HTTPS requires Apache and a certificate.
6) Proxy relies on correct routing/DNS and must be opened in the firewall.
7) Firewall comes last so you do not block yourself while installing services.

# 4. Switch Configuration (VLANs + Access Ports + Trunk)

4.1 Create VLANs on the switch:

enable configure terminal vlan 10 name USERS_A vlan 20 name USERS_B vlan 99 name SERVERS end write memory

4.2 Assign access ports (PCs and Server):

configure terminal interface fa0/2 switchport mode access switchport access vlan 10 spanning-tree portfast exit interface fa0/3 switchport mode access switchport access vlan 20 spanning-tree portfast exit interface fa0/10 switchport mode access switchport access vlan 99 spanning-tree portfast exit end write memory

4.3 Configure trunk port to router (carry multiple VLANs):

configure terminal interface fa0/1 switchport mode trunk switchport trunk allowed vlan 10,20,99 end write memory

4.4 Verify switch VLANs and trunk:

show vlan brief show interfaces trunk

# 5. Router Configuration (Router-on-a-Stick + DHCP Relay)

5.1 Enable the physical interface connected to the switch (example uses G0/0):

enable configure terminal interface g0/0 no shutdown end

5.2 Create subinterfaces with 802.1Q encapsulation and gateway IPs:

configure terminal interface g0/0.10 encapsulation dot1Q 10 ip address 10.10.10.254 255.255.255.0 exit interface g0/0.20 encapsulation dot1Q 20 ip address 10.10.20.254 255.255.255.0 exit interface g0/0.99 encapsulation dot1Q 99 ip address 10.10.99.254 255.255.255.0 exit end write memory

5.3 DHCP relay (ip helper-address) for VLAN10 and VLAN20 (DHCP server is 10.10.99.10):

configure terminal interface g0/0.10 ip helper-address 10.10.99.10 exit interface g0/0.20 ip helper-address 10.10.99.10 exit end write memory

5.4 Verify router interfaces and addressing:

show ip interface brief show running-config interface g0/0.10 show running-config interface g0/0.20 show running-config interface g0/0.99

# 6. Ubuntu Server: Static IP (VLAN99)

Set a static IP for the server so clients and routers can always find DHCP/DNS/Web/Proxy.

6.1 Check interface name:

ip a

6.2 Edit netplan (example interface name: ens33). Adjust to your NIC name:

sudo nano /etc/netplan/01-netcfg.yaml # Example: network: version: 2 renderer: networkd ethernets: ens33: dhcp4: no addresses: [10.10.99.10/24] gateway4: 10.10.99.254 nameservers: addresses: [127.0.0.1, 8.8.8.8]

6.3 Apply and verify:

sudo netplan apply ip a ip route ping -c 3 10.10.99.254

# 7. DHCP Server (isc-dhcp-server) for VLAN10 and VLAN20

7.1 Install DHCP server:

sudo apt update sudo apt install isc-dhcp-server -y

7.2 Bind DHCP to the correct interface:

sudo nano /etc/default/isc-dhcp-server # Set (example): INTERFACESv4="ens33"

7.3 Configure DHCP scopes:

sudo nano /etc/dhcp/dhcpd.conf default-lease-time 600; max-lease-time 7200; authoritative; option domain-name "lab.local"; option domain-name-servers 10.10.99.10; subnet 10.10.99.0 netmask 255.255.255.0 { option routers 10.10.99.254; } subnet 10.10.10.0 netmask 255.255.255.0 { range 10.10.10.100 10.10.10.200; option routers 10.10.10.254; option broadcast-address 10.10.10.255; option domain-name-servers 10.10.99.10; option domain-name "lab.local"; } subnet 10.10.20.0 netmask 255.255.255.0 { range 10.10.20.100 10.10.20.200; option routers 10.10.20.254; option broadcast-address 10.10.20.255; option domain-name-servers 10.10.99.10; option domain-name "lab.local"; }

7.4 Start and verify DHCP service:

sudo systemctl enable isc-dhcp-server sudo systemctl restart isc-dhcp-server sudo systemctl status isc-dhcp-server --no-pager sudo journalctl -u isc-dhcp-server -n 50 --no-pager

Client test: set PC NIC to DHCP and confirm:

VLAN10 client gets 10.10.10.100-200, gateway 10.10.10.254, DNS 10.10.99.10

VLAN20 client gets 10.10.20.100-200, gateway 10.10.20.254, DNS 10.10.99.10

# 8. DNS Server (BIND9) for lab.local

8.1 Install BIND9 and tools:

sudo apt install bind9 bind9utils dnsutils -y

8.2 Configure forwarders (so your DNS can resolve internet names too):

sudo nano /etc/bind/named.conf.options # Inside options { ... } add: forwarders { 8.8.8.8; 1.1.1.1; }; allow-query { any; }; recursion yes;

Restart DNS:

sudo systemctl restart bind9

8.3 Define your zone:

sudo nano /etc/bind/named.conf.local zone "lab.local" { type master; file "/etc/bind/db.lab.local"; };

8.4 Create the zone file:

sudo cp /etc/bind/db.local /etc/bind/db.lab.local sudo nano /etc/bind/db.lab.local $TTL 604800 @ IN SOA dns.lab.local. admin.lab.local. ( 2 ; Serial 604800 ; Refresh 86400 ; Retry 2419200 ; Expire 604800 ) ; Negative Cache TTL @ IN NS dns.lab.local. dns IN A 10.10.99.10 www IN A 10.10.99.10 proxy IN A 10.10.99.10

8.5 Validate and test DNS:

sudo named-checkconf sudo named-checkzone lab.local /etc/bind/db.lab.local sudo systemctl restart bind9 dig @127.0.0.1 www.lab.local dig @127.0.0.1 google.com

# 9. Apache Web Server (HTTP + HTTPS)

9.1 Install and enable Apache:

sudo apt install apache2 -y sudo systemctl enable apache2 sudo systemctl status apache2 --no-pager

9.2 Create a website directory and a test page:

sudo mkdir -p /var/www/lab echo "<h1>It works: www.lab.local</h1>" ¦ sudo tee /var/www/lab/index.html

9.3 Create a VirtualHost for HTTP (port 80):

sudo nano /etc/apache2/sites-available/lab.conf <VirtualHost *:80> ServerName www.lab.local DocumentRoot /var/www/lab <Directory /var/www/lab> AllowOverride All Require all granted </Directory> ErrorLog ${APACHE_LOG_DIR}/lab_error.log CustomLog ${APACHE_LOG_DIR}/lab_access.log combined </VirtualHost>

Enable site and reload Apache:

sudo a2ensite lab.conf sudo a2dissite 000-default.conf sudo systemctl reload apache2

Test from server (and later from clients):

curl -I http://www.lab.local

9.4 Enable HTTPS with a self-signed certificate (lab use):

sudo a2enmod ssl sudo systemctl restart apache2 sudo openssl req -x509 -nodes -days 365 -newkey rsa:2048 -keyout /etc/ssl/private/lab.key -out /etc/ssl/certs/lab.crt # When asked for Common Name (CN), enter: www.lab.local

Create SSL VirtualHost (port 443):

sudo nano /etc/apache2/sites-available/lab-ssl.conf <VirtualHost *:443> ServerName www.lab.local DocumentRoot /var/www/lab SSLEngine on SSLCertificateFile /etc/ssl/certs/lab.crt SSLCertificateKeyFile /etc/ssl/private/lab.key ErrorLog ${APACHE_LOG_DIR}/lab_ssl_error.log CustomLog ${APACHE_LOG_DIR}/lab_ssl_access.log combined </VirtualHost>

Enable SSL site and reload:

sudo a2ensite lab-ssl.conf sudo systemctl reload apache2

Optional: redirect HTTP to HTTPS:

sudo a2enmod rewrite sudo nano /etc/apache2/sites-available/lab.conf # Add inside <VirtualHost *:80>: # Redirect permanent / https://www.lab.local/ sudo systemctl reload apache2

Test HTTPS (ignore cert warning in lab):

curl -kI https://www.lab.local

# 10. Squid Proxy (policy + caching)

10.1 Install Squid:

sudo apt install squid -y sudo systemctl enable squid

10.2 Configure Squid (basic policy example):

sudo nano /etc/squid/squid.conf http_port 3128 acl localnets src 10.10.10.0/24 10.10.20.0/24 acl Safe_ports port 80 443 acl CONNECT method CONNECT acl blocked_sites dstdomain "/etc/squid/blocked.txt" acl office_hours time MTWHF 08:00-17:00 http_access deny !Safe_ports http_access deny CONNECT !Safe_ports http_access deny blocked_sites office_hours http_access allow localnets http_access deny all

Create a blocked domains list and restart Squid:

echo ".facebook.com" ¦ sudo tee /etc/squid/blocked.txt echo ".youtube.com" ¦ sudo tee -a /etc/squid/blocked.txt sudo squid -k parse sudo systemctl restart squid sudo systemctl status squid --no-pager

Client proxy settings:

Proxy IP: 10.10.99.10

Port: 3128

Test browsing and check /var/log/squid/access.log on the server.

# 11. Firewall (UFW on Ubuntu Server)

Configure firewall after services are installed so you avoid blocking setup steps.

sudo ufw default deny incoming sudo ufw default allow outgoing # SSH (admin) from internal networks sudo ufw allow from 10.10.0.0/16 to any port 22 proto tcp # DHCP sudo ufw allow 67/udp # DNS sudo ufw allow 53 sudo ufw allow 53/tcp sudo ufw allow 53/udp # Web sudo ufw allow 80/tcp sudo ufw allow 443/tcp # Squid sudo ufw allow 3128/tcp sudo ufw enable sudo ufw status verbose

# 12. Final End-to-End Testing Checklist

Run these in order. Do not skip - each step proves one layer works.

Switch: show vlan brief and show interfaces trunk (confirm VLANs and trunk).

Router: show ip interface brief (confirm subinterfaces are up).

Clients: obtain DHCP address (correct subnet, gateway, DNS).

From clients: ping gateway (10.10.10.254 or 10.10.20.254).

From clients: ping server (10.10.99.10).

From clients: nslookup www.lab.local (must return 10.10.99.10).

From clients: open http://www.lab.local (should show the test page).

From clients: open https://www.lab.local (accept warning if self-signed).

Proxy: set browser proxy to 10.10.99.10:3128 and test allowed/blocked sites.

Firewall: confirm ufw status and that services remain reachable.

# 13. Troubleshooting Quick Reference (Most Common Issues)

| Symptom | Most likely cause | Fix / command |
| --- | --- | --- |
| Clients do not get DHCP IP | Missing ip helper-address or VLAN/trunk mismatch | Router: verify ip helper-address on g0/0.10 and g0/0.20; Switch: show interfaces trunk; Server: systemctl status isc-dhcp-server |
| DNS fails (SERVFAIL) or no zone answers | Zone file syntax/serial issue or bind not restarted, firewall blocking 53 | named-checkconf; named-checkzone; systemctl restart bind9; ufw allow 53 |
| Web works on server but not from clients | Firewall blocking 80/443 or routing/DNS wrong | From client: ping 10.10.99.10; curl http://10.10.99.10; ufw allow 80/443 |
| HTTPS name mismatch warning | Certificate CN not www.lab.local | Re-create cert with CN=www.lab.local and reload Apache |
| Proxy not working | Squid ACL order or firewall blocking 3128 | squid -k parse; check http_access order; ufw allow 3128; tail -f /var/log/squid/access.log |

End of guide.